# Part B, ANNEX II + III:	TECHNICAL SPECIFICATIONS + TECHNICAL OFFER / TEHNIČKE SPECIFIKACIJE I TEHNIČKA PONUDA

Contract title / Naziv ugovora: Supply of Drying system on biomass energy source/ Nabavu  opreme za Sustav za sušenje na bio-masu

Publication reference / Broj natječaja: IPA2007/HR/16IPO/001-040402/SUP1

Column 1-2 should be completed by the Contracting Authority / Kolone 1-2 treba ispuniti Ugovarateljno tijelo

Column 3-4 should be completed by the tenderer/ Kolone 3-4 treba ispuniti ponuditelj

Column 5 is reserved for the evaluation committee / Kolona 5 je rezervirana za evaluacijski odbor

Annex III – the Contractor’s technical offer

The tenderers are requested to complete the template on the next pages: / Natjecatelji su dužni ispuniti obrazac na slijedećim stranicama:

- Column 2 is completed by the Contracting Authority and it shows the required specifications (not to be modified by the tenderer); / Kolonu 2 ispunjava Ugovarateljno tijelo sa specifikacijama tražene robe (natjecatelji ih ne smiju modificirati)

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient); / Kolonu 3 treba ispuniti natjecatelj detaljnim opisom robe koju nudi (riječi kao “sukladan” ili “da” nisu dovoljne)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation / U koloni 4 natjecatelji su slobodni dodati svoje komentare i eventualna dodatna objašnjenja odnosno reference na prateće dokumente

- Column 5 is reserved for the Evaluation Committee and should not be modified/filled by the tenderer / Kolona 5 je rezervirana za evaluacijski odbor i ne smije biti mijenjana / ispunjena od strane natjecatelja

The eventual supporting documentation supplied in addition to this technical offer should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee. / Eventualna prateća dokumentacija koja se dostavi kao nadopuna ovoj ponudi mora jasno ukazivati na modele odnosno opcije koje se nude, kako bi evaluatori jasno mogli razaznati konfiguraciju / ponudu. Ponude koje ne identificiraju precizno modele i specifikacije mogu biti odbijeni od strane evaluacijskog odbora.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications./ Ponuda mora biti dovoljno jasna da omogući evaluatorima jednostavnu usporedbu između traženih i ponuđenih specifikacija.

Technical specifications provided in the table below are in the format of a checklist, covering the equipment, implementation tasks and after sales service. They are compulsory as a minimum standard for each item of equipment and will be the only basis for the evaluators to assess the technical compliance of the equipment presented in the offers / Tehničke specifikacijenavedene su u tablici u formatu check- liste koja obuhvaća opremu, zadatke provedbe i post-prodajne usluge. One su obavezne kao minimalni standard svakepojedine stavke tražene robe i jedina su osnova za ocjenu tehničke sukladnosti ponuda od strane evaluatora.

TECHNICAL SPECIFICATIONS– EQUIPMENT / TEHNIČKESPECIFIKACIJE - OPREMA

| I Item No. / br. stavke | II Specifications Required / Tražene specifikacije |  | III Specifications Offered / Ponuđene specifikacije | IV Notes, remarks, ref to documentation / Bilješke, napomene, reference nadokumentaciju |  | V Evaluation Committee’s notes / Bilješkeevaluacijskogodbora |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | LOT1 - Pumpkin seed drying preparation line line / Linija pripreme bućinih sjemenki za sušenje (quantity: 1 line/1 linija) |  |  |  |  |  |
| 1.1. | Kontejner za prihvat sirovih sjemenki – 2 komada / Container for the reception of raw seeds - 2 pieces |  |  |  |  |  |
|  | Parameter | Characteristics (minimal requirement) |  |  |  |  |
|  | Material /Materijal | Čelični lim / Steel sheet |  |  |  |  |
|  | Capacity / Kapacitet | 2 tone / 2 tones |  |  |  |  |
|  | Form / Oblik | It should be rectangular in shape, on the front should be conical placed obliquely outlet with sliding lock to release the seeds; / Treba biti pravokutnog oblika, na prednjoj strani treba imati konusni koso postavljeni ispust sa kliznim zatvaračem za ispuštanje sjemenki; |  |  |  |  |
| 1.2. | Washing device of raw seeds - 1 piece / Uređaj za pranje sirovih sjemenki – 1 komad |  |  |  |  |  |
|  | Description / Opis | The device consists of a hopper, drum unit made ​​of perforated sheet metal and electric motor driven by sprockets. The drum on the inside must be placed with washers of seeds and on the outside must be placed with nozzles with rotating brushes to wash perforated sheet. The drum should be placed obliquely to fall towards the exit of seeds, along washer must be located a rack for draining of washed seeds; / Uređaj se sastoji od usipnog koša, bubnja od perforiranog lima i elektro motornim pogonom putem lančanika. U bubnju sa unutarnje strane se moraju nalaziti mlaznice za pranje sjemenki, sa vanjske strane se moraju nalaziti mlaznice sa rotacionim četkama koje peru perforirani lim. Bubanj mora biti koso postavljen sa padom prema izlazu sjemenki, uz peračicu se mora nalaziti i stalak za ocjeđivanje opranih sjemenki; |  |  |  |  |
| 1.3. | Container for wet seeds with a track for charging the dryer - 1 piece / Kontejner za vlažne sjemenke sa trakom za punjenje sušare – 1 komad |  |  |  |  |  |
|  | Material /Materijal | Inox sheet / Inox lim |  |  |  |  |
|  | Description / Opis | Track for charging the dryer must satisfy dryer capacity / Traka za punjenje sušare mora zadovoljiti kapacitet sušare. |  |  |  |  |
| 1.4 | Track for emptying the dryer with a rough cleaner with aspiration / Traka za pražnjenje sušare sa grubim prečistaćem sa aspiracijom |  |  |  |  |  |
|  | Description / Opis | Track for emptying the dryer with a rough cleaner with aspiration / Traka za pražnjenje sušare sa grubim prečistaćem sa aspiracijom |  |  |  |  |
| 2 | LOT2 - Drying lines / Linije za sušenje (quantity: 2 lines/2 linije) |  |  |  |  |  |
| 2.1. | Automatic heater - 2 pieces / Automatski termogen – 2 komada |  |  |  |  |  |
|  | Parameter | Characteristics (minimal requirement) |  |  |  |  |
|  | Power / Snaga | 450 kW |  |  |  |  |
|  | Description / Opis | Automatic heater that gets its energy from biomass of corncobs corn, seeds of plums, cherries, etc., and shells of walnuts, hazelnuts, kakoa etc. should have a system for automated dispensing and burning of mentioned fuel, tank for ash removal with the cochlea for the ash, control panel with an automatic fan for air blowing. / Automatski termogen koji dobiva energiju iz biomase oklaska kukuruza, sjemenki šljiva, višanja i dr. te ljusaka od oraha, lješnjaka, kakoaoca i sl. Treba imati sistem za automatsko doziranje i sagorijevanje navedenog goriva, rezervoar za pepeo sa pužnicama za iznošenje istog, komandni ormar sa automatikom i ventilator za upuhivanje zraka. |  |  |  |  |
| 2.2. | Radijalni ventilator – 2 komada |  |  |  |  |  |
|  | Capacity / Kapacitet | from 30.000 to 40.000 m3/hour / od 30.000 do 40.000 m3/sat |  |  |  |  |
|  | Description / Opis | It consists of input and output channels and is powered by an electric motor of power 7-8 kW, and from 2500 to 2800 rpm. / Sastoji se od ulaznog i izlaznog kanala a pokreće ga elektromotor snage od 7 do 8 kW i od 2500 do 2800 okretaja u minuti. |  |  |  |  |
| 2.3. | Space for drying pumpkin seeds - 2 pieces / Prostor za sušenje bućinih sjemenki – 2 komada |  |  |  |  |  |
|  | Material /Materijal | Steel sheet and profiles / Čelični lim i profili |  |  |  |  |
|  | Dimensions / Veličina | Surface for drying of 20m2 / Površina za sušenje od 20m2 |  |  |  |  |
|  | Description / Opis | Drying area of pumpkin seeds consisting of two parts: the lower part is a channel for the supply and distribution of hot air and the upper part on which are wet seeds placed for drying. Floor of the dryer should be made ​​of perforated steel sheet thickness of 2-3 mm and with perforation diameters of 8 mm. The amount of the charge should be from 5 to 10 m3. / Prostor za sušenje bućinih sjemenki koji se sastoji iz dva dijela: donji dio čini kanal za dovod i raspodjelu toplog zraka te gornji dio na koji se nanose vlažne sjemenke za sušenje. Pod sušare treba biti od perforiranog čelika debljine lima od 2 do 3 mm i promjera rupica 8 mm. Količina šarže treba biti od 5 do 10 m3. |  |  |  |  |
| 2.4. | Material hopper with cochlea for transport of biomass - 2 pieces / Usipni koš sa pužnicom za transport bio mase – 2 komada |  |  |  |  |  |
|  | Material /Materijal | Steel sheet / Lim |  |  |  |  |
|  | Volume / Volumen | 4m3 |  |  |  |  |
|  | Description / Opis | Material hopper with cochlea for transport of biomass and fine dosing with length of 5 m and with diameter of 250 mm; / Usipni koš sa pužnicom za transport bio mase i fino doziranje dužine 5 m i promjera 250 mm; |  |  |  |  |
| 2.5. | Mixer of seeds bidirectional - 2 pieces / Mješač sjemenki dvosmjerni – 2 komada |  |  |  |  |  |
|  | Material /Materijal | Steel tubes and profiles / Čelične cijevi i profili |  |  |  |  |
|  | Description / Opis | Mixer of seeds and suction hood located on the carrier and driven by two electric motors with gear reduction units. Suction hood that moves with mixer and sucks moisture and glumes should be made of inox steel sheet. Vacuuming should be done by axial fan with power of 3 kW. / Mješač sjemenki i usisna hauba koji se nalaze na nosaču a pokreću ih dva elektromotora sa reduktorima. Usisna hauba koja se pomiče sa mješačem i usisava vlagu i pljevicu treba biti od inox lima. Usisavanje vrši aksijalni ventilator snage 3 kW. |  |  |  |  |
| 2.6. | Chimney - 2 pieces / Dimnjak – 2 komada |  |  |  |  |  |
|  | Dimensions / Dimenzije | 800mm in diameter, total height of 7 meters / Promjer 800mm, ukupne visine 7 metara |  |  |  |  |
|  | Material /Materijal | Sheet metal / Lim |  |  |  |  |
|  | Description / Opis | Must include a gate for controling and cleaning, collector of sparks and on the top must have a protective cap. / Mora sadržavati vrata za kontrolu i čišćenje, sakupljač iskre te na vrhu mora imati zaštitnu kapu. |  |  |  |  |
| 2.7. | Smoke tube - 2 pieces / Dimna cijev – 2 komada |  |  |  |  |  |
|  | Dimensions / Dimenzije | 400 mm in diameter, total length of 6-7 meters / Promjer 400 mm, ukupne dužine od 6 do 7 metara |  |  |  |  |
|  | Material /Materijal | sheet metal / lim |  |  |  |  |
|  | Description / Opis | The smoke tubes connects heater to a chimney. On the smoke tube must be installed cyclone for collecting of ash and red-hot parts of the bio-fuel. / Dimnom cijevi se spaja termogen sa dimnjakom. Na dimnoj cijevi mora biti ugrađen ciklon za sakupljanje pepela i užarenih dijelova bio goriva. |  |  |  |  |
| 2.8. | Small material hopper - 2 pieces / Mali usipni koš – 2 komada |  |  |  |  |  |
|  | Capacity / Kapacitet | 1m3 |  |  |  |  |
|  | Description / Opis | For bio fuel placed next to the heater and equipped with cochlea for fine dosing of fuel in the heater. The electric motor that drives the cochlea must be connected to the automatic control which regulates and maintains the set temperature of the drying air. Due to the security of returning fire there must be installed a transfer points cochlea. / Za bio gorivo smješten uz termogen i opremljen pužnicom za fino doziranje goriva u termogen. Elektromotor koji pokreće pužnicu mora biti spojen na automatiku koja regulira i održava zadanu temperaturu zraka za sušenje. Zbog sigurnosti od povratka vatre potrebno je ugraditi presipnu pužnicu. |  |  |  |  |
| 3 | LOT 3: Pumpkin oil mill – 1 line / Uljara – 1 linija |  |  |  |  |  |
| 3.1. | Cyclone with precleaner for lifting of seeds in a mill for grinding - 1 piece / Ciklon sa prečistačem za podizanje sjemenki u mlin za meljavu – 1 komad |  |  |  |  |  |
|  | Dimensions / Dimenzije | Height of up to 5 meters / Visina do 5 metara |  |  |  |  |
|  | Description / Opis | Raises seeds to filters that separate dirt and then lead seeds to the mill for fine crushing, must be made of Inox sheet W.Nr.1.4301 / Podiže sjemenke do prečistača koji odvaja nečistoće a sjemenke dovodi do mlina za fino usitnjavanje sjemenki,mora biti izrađen od inox lima W.Nr.1.4301 |  |  |  |  |
| 3.2. | Conditioner - 1 piece / Kondicioner – 1 komad |  |  |  |  |  |
|  | Material /Materijal | Inox sheet W.Nr.1.4301 / Inox lim W.Nr.1.4301 |  |  |  |  |
|  | Description / Opis | It serves to prepare the milled seeds, located below the cyclone and mill. Conditioner should be equipped with a mixer of seeds and must be integrated into vertical lines of the entire plant. / Služi za pripremu samljevenih sjemenki, nalazi se ispod ciklona i mlina. Kondicioner treba biti opremljen sa mješačem sjemenki i mora biti uklopljen u vertikalnu izvedbu linije cijelog postrojenja. |  |  |  |  |
| 3.3. | The boiler for baking - 1 piece / Kotao za pečenje – 1 komad |  |  |  |  |  |
|  | Material /Materijal | Insulated and covered with inox sheet W.Nr.1.4301 / Izoliran i obložen inox limom W.Nr.1.4301 |  |  |  |  |
|  | Description / Opis | The boiler for baking milled and conditioned mixture with a gas burner and automatic operation in the desired conditions. The base of the boiler should be made ​​in three layers of a total thickness of 25 mm with minimal losses. Above the boiler needs to be mixer powered by reducer on a movable console. / Kotao za pečenje samljevene i kondicionirane smjese sa plinskim plamenikom i automatskim radom u željenim uvjetima. Podnica kotla treba biti izrađena u tri sloja ukupne debljine 25 mm uz minimalne gubitke. Iznad kotla treba biti mješač pogonjen reduktorom na pomičnoj konzoli. |  |  |  |  |
| 3.4. | Suction hood - 1 piece / Usisna napa - 1 komad |  |  |  |  |  |
|  | Material /Materijal | Inox sheet W.Nr.1.4301 / Inox lim W.Nr.1.4301 |  |  |  |  |
|  | Description / Opis | For vacuuming of steam and must be located above the boiler to drain off water vapor out of operation. Suction through axial fan driven by electric power of 1 kW with two speeds. / Za usisavanje pare i mora biti smještena iznad kotla te odvoditi vodenu paru izvan pogona. Usisavanje preko aksijalnog ventilatora kojeg pokreće elektromotor snage 1 kW sa dvije brzine. |  |  |  |  |
| 3.5 | The press for pumpkin seed oil - 1 piece / Preša za bučino ulje – 1 komad |  |  |  |  |  |
|  | Capacitiy / Kapacitet | From 25 to 30 kilograms of a mixture / Od 25 do 30 kilograma smjese |  |  |  |  |
|  | Description / Opis | Hydraulic press for pressing at pressure calendering of minimum 380 bar. Engine power of hydro pump of 4 kW / Hidraulična preša za prešanje pri tlaku tiješnjenja minimalno 380 bara. Snaga motora hidro pumpe 4 kW. |  |  |  |  |
| 3.6. | Reservoir for deposition - 1 piece / Rezervoar za taloženje – 1 komad |  |  |  |  |  |
|  | Material /Materijal | Inox sheet W.Nr.1.4301 / Inox lim W.Nr.1.4301 |  |  |  |  |
|  | Volume / Volumen | 600 litres / 600 litara |  |  |  |  |
|  | Description / Opis | The vertical version, with control hole, level indicators, a lid, drain valve, testing faucet and thermometer. / U stojećoj izvedbi, sa kontrolnim otvorom, nivokazom, poklopcem, izljevnim ventilom, slavinom za probu i termometrom. |  |  |  |  |
| 3.7. | Container - 1 piece / Spremnik – 1 komad |  |  |  |  |  |
|  | Material /Materijal | Inox sheet W.Nr.1.4301 / Inox lim W.Nr.1.4301 |  |  |  |  |
|  | Volume / Volumen | 2500 litres / 2500 litara |  |  |  |  |
|  | Description / Opis | In the standing version for pumpkin oil storage before packaging. The container must have an opening of 300 * 400 mm for cleaning, faucet for oil dispensing, connector for thermometer and control opening of Ø 200 mm / U stojećoj verziji za skladištenje bućinog ulja prije pakiranja. Na spremniku mora biti otvor 300*400 mm zbog čišćenja, slavina za istakanje ulja, priključak za termometar i kontrolni otvor fi 200 mm. |  |  |  |  |
| 3.8. | Filling machine - 1 piece / Punilica – 1 komad |  |  |  |  |  |
|  | Capacity / Kapacitet | 1000 pieces /h / 1000 komada /h |  |  |  |  |
| 3.9. | Labeling machine - 1 piece / Etiketirka - 1 komad |  |  |  |  |  |
|  | Capacity / Kapacitet | 1000 pieces /h / 1000 komada /h |  |  |  |  |
| 3.10. | The Bottle shutter - 1 piece / Zatvarač boca – 1 komad |  |  |  |  |  |
|  | Capacity / Kapacitet | 1000 pieces /h / 1000 komada /h |  |  |  |  |

TECHNICAL SPECIFICATIONS – IMPLEMENTATION TASKS / TEHNIČKE SPECIFIKACIJE – PROVEDBENI ZADACI - APPLICABLE TO ALL LOTS

| I Item no / br. stavke | II Implementation tasks /Provedbeni zadaci | III Specifications Offered (please specify the duration, resources and methodology proposed / Ponuđene specifikacije (molimo specificirajte trajanje, resurse i metodologiju) | IV Notes, remarks, ref to documentation / Bilješke, napomene, reference nadokumentaciju | V Evaluation Committee’s notes / Bilješke evaluacijskog odbora |
| --- | --- | --- | --- | --- |
| 1. | Task 1 Delivery of equipment / Dostava opreme Delivery shall be made at the latest after 90 days/s after the commencement of the Contract. / Dostava će se izvršiti najkasnije 90 dana od početka provedbe ugovora; |  |  |  |
| 2. | Task 2 Installation of equipment / Instalacija opreme Installation of equipment shall be made at company Grbić Ltd. premises at Mlinska bb, 34000 Pozega, Croatia / Instalacija opreme biti će izvršena u prostorima tvrtke Grbić d.o.o., Mlinska bb, 34000 Pozega, Hrvatska; |  |  |  |

TECHNICAL SPECIFICATIONS – AFTER-SALES SERVICES / TEHNIČKESPECIFIKACIJE-  POST-PRODAJNEUSLUGE

| I Item no. / Br. stavke | II After sales services / Usluge | III Specifications Offered (please specify the duration, resources and methodology proposed / Ponuđenespecifikacije (molimospecificirajtetrajanje, resurseimetodologiju) | IV Notes, remarks, ref to documentation / Bilješke, napomene, reference nadokumentaciju | V Evaluation Committee’s notes / Bilješkeevaluacijskogodbora |
| --- | --- | --- | --- | --- |
| 1. | Warranty / Garancija Warranty on installed materials and assemblies is 3 years after provisional acceptance/ Garantni rok na ugrađene materijale i sklopove je 3 godine nakon privremenog prihvata robe; |  |  |  |
| 2. | Servicing and avaliablity of spare parts for 5 years after the provisonal acceaptance. / Servis i dostupnost rezervnih dijelova 5 godina nakon privremenog prihvata robe. |  |  |  |
| 3. | Training for maintanance / Trening za održavanje opreme Training of 4 Grbić Ltd. employees in maintanance of newly installed equipment. / Trening za 4 djelatnika tvrtke Grbić d.o.o. o održavanju novo instalirane opreme. |  |  |  |

| Authorised contact person / Ovlaštena kontakt osoba |  | Delivery address / Adresa dostave |
| --- | --- | --- |
| Name: |  | <name of the institution / ime organizacije> <address / adresa> |
| Phone: |  |  |
| Fax: |  |  |
| E-mail: |  |  |
| Working hours: |  |  |

ANNEX 1-  EQUIPMENT ORIGIN STATEMENT / IZJAVA O PORIJEKLU ROBE

Contract title / Naziv ugovora: Supply of Drying system on biomass energy source/ Nabavu  opreme za Sustav za sušenje na bio-masu

Publication reference / Broj natječaja: IPA2007/HR/16IPO/001-040402/SUP1

Please find our expressed statement that all supplies, equipment and services to be supplied unless otherwise provided in the Special Conditions, originate in a Member State of the European Union, European Economic Area, candidate or potential candidate country or a country or territory of the regions covered and/or authorised by the IPA Regulation ((EC) no. 1085/2006 of 17 July 2006). / Ovime izričito izjavljujemo da sva roba, oprema i usluge koje se dostavljaju, osim ako je drugačije definirano Posebnim uvjetima, potječu iz zemlje članice EU, članice EEA, zemlje kandidatkinje ili potencijalne kandidatkinje, te države ili teritorija koji se smatra prihvatljivim prema IPA regulativi (br 1085/2006 od 17. srpnja 2006).

The term "origin" is defined in the relevant international agreements (notably WTO agreements), which is reflected in EU legislation on rules of origin for customs purposes (i.e. Council Regulation (EEC) No. 2913/92 (OJ L302 of 19.10.92) – “the EU Customs Code”) and its implementing provisions (Commission Regulation (EEC) No. 2454/93 (OJ L253 of 11.10.93). The corresponding jurisprudence of the Court of Justice of the European Communities must also be taken into account.  / Pojam 'porijeklo' je definiran relevatnim međunarodnim sporazumima (prvenstveno sporazumima Svjetske trgovinske organizacije), što je reflektirano u EU zakonu o pravilima porijekla za potrebe carinjenja (tj Council Regulation (EEC) No. 2913/92 (OJ L302 iz 19.10.1992) – “the EU Customs Code”), i provedbenoj regulativi (Commission Regulation (EEC) No. 2454/93 (OJ L253 of 11.10.93). Relevantna zakonska tumačenja Suda Europske unije također treba uzeti u obzir.

| Item No. | Equipment / Roba | Country of Origin** / Državaporijekla** |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |

**The usage of the expression “European Community”, “European Union”, “EU” or “IPA” is not acceptable here. The specific country, whose relevant organisation will issue the relevant supporting documentation on Origin for the manufacture of the goods, is required to be inserted here. / Korištenje izraza kao što su 'Europska zajednica', 'Europska unija'm 'EU' ili 'IPA' nije ovdje prihvatljivo. Konkretna država, čija relevantna organizacija će izdati pripadajuće dokazne dokumente o Porijeklu robe, mora biti navedena ovdje.

Done at:[……place/mjesto…],    Date / datum:[../../..]		By:[… name / ime …]

On behalf of:[……Tenderer’s name/ Naziv ponuditelja……]	[tenderer's stamp and signature]

If appropriate, prepare a separate table on the next page, specifying  the schedule and place(s) of delivery (e.g. in cases of complex/multiple delivery on several addresses)/ Ako je prikladno, pripremiti na ovojenom listu posebnu tablicu sa specifikacijom mjesta i vremena dostave (npr. u slučajevima kompleksne/višestrane dostave na različite adrese)

This statement should be included in part 3 of tenderer’s offer (see Article 9 of Instruction to tenderers). Tenderers are obliged [free ] to use this template, [ or to testify to the origin of supplies in other format of their choice]. / Ova izjava treba biti djelom 3 ponude natjecatelja (vidi članak 9. Uputa natjecateljima). Ponuditelji su obavezni [slobodni] koristiti ovaj format, [ili dokazati podrijetlo opreme u formatu po vlastitom izboru].